CT LSU Load Address Generation

Verification Specification

Module: ct_lsu_ld_ag
Version: 1.0
Status: RTL-derived draft for verification review
Date: 2026-07-11

| Item | Value |
| --- | --- |
| RTL file | ct_lsu_ld_ag.v |
| RTL SHA-256 | 7c62256e615d66ed54bf96e10373e2636206832cc33d5749e789257f9bb8a010 |
| Physical lines | 1462 |
| Upstream baseline | https://github.com/XUANTIE-RV/openc910/blob/main/C910_RTL_FACTORY/gen_rtl/lsu/rtl/ct_lsu_ld_ag.v |
| Important limitation | Standalone module only; adjacent-stage timing is inferred from signal usage and must be confirmed by design. |

# Document Control

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-11 | Codex | Initial RTL-derived verification specification |

# Table of Contents

1. Purpose and Scope

2. Module Overview

3. Clock, Reset, and Pipeline Timing

4. Interface Specification

5. Functional Requirements

6. Configuration Matrix

7. Verification Strategy

8. Suggested Assertions

9. Coverage Model

10. Open Questions and Assumptions

# 1. Purpose and Scope

This document specifies observable behavior of the load address-generation (LD AG) stage for block-level and subsystem verification. It is derived from the supplied RTL, not from a separate architecture document. Therefore, each statement is classified as an RTL fact, an inferred interface contract, or an open question.

## 1.1 In Scope

RF-to-AG instruction capture and stall retention.

Virtual-address calculation for immediate and register-offset loads.

Natural-alignment, 16-byte boundary, and 4 KiB page-crossing detection.

Byte masks, data rotation, DCACHE bank enables, and tag/data indices.

MMU port0 request/abort behavior and PA construction.

Exception, atomic commit, stall/restart, LSIQ, IDU timing, and performance outputs.

Reset, clock-gating, invalid-state, and X-propagation expectations.

## 1.2 Out of Scope

Correctness of the generic gated_clk_cell and ct_rtu_compare_iid implementations.

DC, DA, WB, MMU, DCACHE, RTU, and IDU behavior except where their interface contract is required to verify AG.

Vector mask generation removed/tied off in this RTL version.

Performance targets, physical timing closure, and power sign-off.

# 2. Module Overview

The module accepts a load-like instruction from IDU/RF pipe3. Control valid is updated on ctrl_ld_clk, while instruction data is captured on a locally gated clock derived from forever_cpuclk. Address generation calculates an effective VA, detects 16-byte and 4 KiB crossings, requests page translation for the base page, constructs a PA, drives speculative DCACHE array reads, and decides whether the instruction advances to DC or restarts/stalls.

RF pipe3 -> AG registers -> VA/size/alignment -> MMU page result -> PA/masks/DCACHE index -> exception & stall arbiter -> DC/IDU/LSIQ

| Block | Primary RTL lines | Responsibility |
| --- | --- | --- |
| Capture and hold | L545-L777 | Capture valid/data, retain on stall, update replay base/offset. |
| VA generation | L789-L810 | Shift offset, select normal/split VA, expose VPN. |
| Size/alignment/boundary | L842-L909 | Decode access size and detect misalignment/16-byte crossing. |
| Masks and banks | L915-L1030, L1145-L1190 | Generate byte masks, rotation, bank enables, and array indices. |
| MMU and PA | L1035-L1068 | Issue base-page translation, consume attributes, build final PA. |
| Exceptions | L1204-L1241 | Generate misalign, page, SO, and atomic cacheability exceptions. |
| Stall/restart | L1247-L1336 | Prioritize atomic, page-cross, DCACHE, and MMU replay conditions. |
| Downstream outputs | L1342-L1456 | Drive DC valid, ahead signals, monitor, LSIQ/IDU, and HPCP events. |

# 3. Clock, Reset, and Pipeline Timing

## 3.1 Clock Domains

| Clock | Registers | Enable/Source | Verification Requirement |
| --- | --- | --- | --- |
| ctrl_ld_clk | ld_ag_inst_vld | External control clock | Flush/reset priority and stall retention shall be checked on every active edge. |
| ld_ag_clk | Instruction attributes, base, offset, replay state | gated_clk_cell from forever_cpuclk | An accepted RF instruction shall cause a data clock edge whenever its valid is captured. |

Open assumption A-CLK-1: ctrl_ld_clk and forever_cpuclk/ld_ag_clk are phase-compatible and use coherent enables. The RTL alone does not prove this relationship.

## 3.2 Reset and Flush Priority

cpurst_b is active low and asynchronously clears ld_ag_inst_vld and most instruction-attribute registers.

rtu_yy_xx_flush synchronously clears ld_ag_inst_vld with higher priority than stall or new issue.

ld_ag_base and ld_ag_offset have no explicit reset. Their values are undefined until a valid RF load is captured; consumers shall qualify dependent outputs with valid.

## 3.3 Nominal Transaction

| Cycle relation | Condition | Expected state/action |
| --- | --- | --- |
| N | idu_lsu_rf_pipe3_sel/gateclk_sel = 1 and no AG stall | RF fields are sampled into AG; AG valid becomes 1. |
| N+1 combinational | AG valid = 1 | VA, masks, MMU request, speculative DCACHE request, exception, and stall logic evaluate. |
| N+1 | No stall/restart | ld_ag_dc_inst_vld = 1 and the instruction advances to DC. |
| N+1 | stall/restart | AG valid/data are retained or replay base/offset are updated according to arbitration. |

# 4. Interface Specification

Widths are taken directly from the module declaration. Unless a signal description states otherwise, outputs are combinational functions of the current AG registers and shall be sampled only with the associated valid.

## 4.1 Clock, Reset, and CP0

| Signal | Dir | Width | RTL line | Description / validity |
| --- | --- | --- | --- | --- |
| cp0_lsu_cb_aclr_dis | input | 1 | 162 | 禁止跨 16-byte 边界的 cache-buffer acceleration。 |
| cp0_lsu_da_fwd_dis | input | 1 | 163 | 禁止 load ahead/DA forwarding 相关有效信号。 |
| cp0_lsu_dcache_en | input | 1 | 164 | 数据缓存使能；门控 tag/data array 请求。 |
| cp0_lsu_icg_en | input | 1 | 165 | LSU 模块时钟门控使能。 |
| cp0_lsu_mm | input | 1 | 166 | misaligned load 处理模式配置；为 0 时普通 load 不对齐在 AG 报错。 |
| cp0_yy_clk_en | input | 1 | 167 | 全局时钟使能。 |
| cpurst_b | input | 1 | 168 | 低有效异步复位。 |
| ctrl_ld_clk | input | 1 | 169 | load 控制时钟，用于更新 AG valid。 |
| forever_cpuclk | input | 1 | 173 | 未门控的 CPU 基准时钟，作为 AG ICG 输入。 |
| pad_yy_icg_scan_en | input | 1 | 212 | 扫描模式下的 ICG 覆盖使能。 |

## 4.2 IDU/RF Inputs

| Signal | Dir | Width | RTL line | Description / validity |
| --- | --- | --- | --- | --- |
| idu_lsu_rf_pipe3_already_da | input | 1 | 174 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_atomic | input | 1 | 175 | 原子类指令标志。 |
| idu_lsu_rf_pipe3_bkpta_data | input | 1 | 176 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_bkptb_data | input | 1 | 177 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_gateclk_sel | input | 1 | 178 | RF pipe3 门控时钟选择；本模块同时把它视为 RF 数据有效。 |
| idu_lsu_rf_pipe3_iid | input | 6:0 | 179 | 指令 ID，用于提交和新旧比较。 |
| idu_lsu_rf_pipe3_inst_fls | input | 1 | 180 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_inst_ldr | input | 1 | 181 | 寄存器 offset 的 LDR 类地址生成标志。 |
| idu_lsu_rf_pipe3_inst_size | input | 1:0 | 182 | 访问大小编码：BYTE/HALF/WORD/DWORD。 |
| idu_lsu_rf_pipe3_inst_type | input | 1:0 | 183 | load/atomic 子类型编码。 |
| idu_lsu_rf_pipe3_lch_entry | input | 11:0 | 184 | 对应 LSIQ entry 的 12-bit one-hot/bitmap。 |
| idu_lsu_rf_pipe3_lsfifo | input | 1 | 185 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_no_spec | input | 1 | 186 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_no_spec_exist | input | 1 | 187 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_off_0_extend | input | 1 | 188 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_offset | input | 11:0 | 189 | 普通 load 的 12-bit 立即数 offset。 |
| idu_lsu_rf_pipe3_offset_plus | input | 12:0 | 190 | split 首拍使用的 13-bit 备选 offset。 |
| idu_lsu_rf_pipe3_oldest | input | 1 | 191 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_pc | input | 14:0 | 192 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_preg | input | 6:0 | 193 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_sel | input | 1 | 194 | RF pipe3 向 AG 发射/选择有效指示。 |
| idu_lsu_rf_pipe3_shift | input | 3:0 | 195 | offset 移位选择，RTL 按 one-hot 编码使用。 |
| idu_lsu_rf_pipe3_sign_extend | input | 1 | 196 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_spec_fail | input | 1 | 197 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_split | input | 1 | 198 | 来自 IDU/RF pipe3 的指令属性或操作数。 |
| idu_lsu_rf_pipe3_src0 | input | 63:0 | 199 | 地址基址源操作数。 |
| idu_lsu_rf_pipe3_src1 | input | 63:0 | 200 | LDR 类指令的 offset 源操作数。 |
| idu_lsu_rf_pipe3_unalign_2nd | input | 1 | 201 | split/边界访问第二拍标志，锁存为 ld_ag_secd。 |
| idu_lsu_rf_pipe3_vreg | input | 6:0 | 202 | 来自 IDU/RF pipe3 的指令属性或操作数。 |

## 4.3 MMU Interface

| Signal | Dir | Width | RTL line | Description / validity |
| --- | --- | --- | --- | --- |
| mmu_lsu_buf0 | input | 1 | 203 | 来自 MMU port0 的翻译结果、属性或流控。 |
| mmu_lsu_ca0 | input | 1 | 204 | 页属性：cacheable。 |
| mmu_lsu_pa0 | input | 27:0 | 205 | MMU 返回的物理页号。 |
| mmu_lsu_pa0_vld | input | 1 | 206 | MMU port0 返回有效；同时限定 CA/SO 和多种异常。 |
| mmu_lsu_page_fault0 | input | 1 | 207 | MMU port0 页故障。 |
| mmu_lsu_sec0 | input | 1 | 208 | 来自 MMU port0 的翻译结果、属性或流控。 |
| mmu_lsu_sh0 | input | 1 | 209 | 来自 MMU port0 的翻译结果、属性或流控。 |
| mmu_lsu_so0 | input | 1 | 210 | 页属性：strong order。 |
| mmu_lsu_stall0 | input | 1 | 211 | MMU stall；上游当前 DUTLB 实现中常为 0，但本模块未按 valid 门控。 |
| lsu_mmu_abort0 | output | 1 | 298 | 中止当前 MMU port0 请求的组合条件。 |
| lsu_mmu_id0 | output | 6:0 | 299 | 发往 MMU port0 的请求、标识或中止信息。 |
| lsu_mmu_st_inst0 | output | 1 | 300 | 发往 MMU port0 的请求、标识或中止信息。 |
| lsu_mmu_va0 | output | 63:0 | 301 | 发往 MMU 的 base VA；页内 offset 在 AG 本地加入。 |
| lsu_mmu_va0_vld | output | 1 | 302 | 发往 MMU port0 的请求有效，等于 AG valid。 |

## 4.4 DCACHE Interface

| Signal | Dir | Width | RTL line | Description / validity |
| --- | --- | --- | --- | --- |
| dcache_arb_ag_ld_sel | input | 1 | 170 | DCACHE arbiter 已选择 load AG 请求；取反形成 cache stall。 |
| dcache_arb_ld_ag_addr | input | 39:0 | 171 | DCACHE arbiter 借用的地址。 |
| dcache_arb_ld_ag_borrow_addr_vld | input | 1 | 172 | DCACHE arbiter 借用地址有效，覆盖下传 DC 地址。 |
| ag_dcache_arb_ld_data_gateclk_en | output | 7:0 | 221 | 发往 DCACHE arbiter 的阵列请求、索引或门控。 |
| ag_dcache_arb_ld_data_high_idx | output | 10:0 | 222 | 发往 DCACHE arbiter 的阵列请求、索引或门控。 |
| ag_dcache_arb_ld_data_low_idx | output | 10:0 | 223 | 发往 DCACHE arbiter 的阵列请求、索引或门控。 |
| ag_dcache_arb_ld_data_req | output | 7:0 | 224 | 发往 DCACHE arbiter 的阵列请求、索引或门控。 |
| ag_dcache_arb_ld_tag_gateclk_en | output | 1 | 225 | 发往 DCACHE arbiter 的阵列请求、索引或门控。 |
| ag_dcache_arb_ld_tag_idx | output | 8:0 | 226 | 发往 DCACHE arbiter 的阵列请求、索引或门控。 |
| ag_dcache_arb_ld_tag_req | output | 1 | 227 | 发往 DCACHE arbiter 的阵列请求、索引或门控。 |

## 4.5 RTU and Store-AG Inputs

| Signal | Dir | Width | RTL line | Description / validity |
| --- | --- | --- | --- | --- |
| rtu_yy_xx_commit0 | input | 1 | 213 | 来自 RTU 的提交端口信息。 |
| rtu_yy_xx_commit0_iid | input | 6:0 | 214 | 来自 RTU 的提交端口信息。 |
| rtu_yy_xx_commit1 | input | 1 | 215 | 来自 RTU 的提交端口信息。 |
| rtu_yy_xx_commit1_iid | input | 6:0 | 216 | 来自 RTU 的提交端口信息。 |
| rtu_yy_xx_commit2 | input | 1 | 217 | 来自 RTU 的提交端口信息。 |
| rtu_yy_xx_commit2_iid | input | 6:0 | 218 | 来自 RTU 的提交端口信息。 |
| rtu_yy_xx_flush | input | 1 | 219 | 全局 flush，清除 AG valid 并中止 MMU。 |
| st_ag_iid | input | 6:0 | 220 | 模块接口信号；具体有效条件由相邻控制信号和功能规则限定。 |

## 4.6 Load-AG and Downstream Outputs

| Signal | Dir | Width | RTL line | Description / validity |
| --- | --- | --- | --- | --- |
| ld_ag_addr1_to4 | output | 35:0 | 228 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_ahead_predict | output | 1 | 229 | load ahead 预测，本版本恒为 1。 |
| ld_ag_already_da | output | 1 | 230 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_atomic | output | 1 | 231 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_boundary | output | 1 | 232 | 普通 load 跨 16-byte 边界或处于第二拍。 |
| ld_ag_dc_access_size | output | 2:0 | 233 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_dc_acclr_en | output | 1 | 234 | 满足 PA valid、cacheable 等条件的边界加速使能。 |
| ld_ag_dc_addr0 | output | 39:0 | 235 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_dc_bytes_vld | output | 15:0 | 236 | 当前 16-byte 数据窗口的字节有效掩码。 |
| ld_ag_dc_bytes_vld1 | output | 15:0 | 237 | 边界访问低地址侧/合并用途的辅助掩码。 |
| ld_ag_dc_fwd_bypass_en | output | 1 | 238 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_dc_inst_vld | output | 1 | 239 | 发往 DC 的有效，排除所有 stall/restart。 |
| ld_ag_dc_load_ahead_inst_vld | output | 1 | 240 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_dc_load_inst_vld | output | 1 | 241 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_dc_mmu_req | output | 1 | 242 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_dc_rot_sel | output | 3:0 | 243 | 基于 VA 低 4 位的数据旋转选择。 |
| ld_ag_dc_vload_ahead_inst_vld | output | 1 | 244 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_dc_vload_inst_vld | output | 1 | 245 | 从 AG 下传到 load DC stage 的数据或控制。 |
| ld_ag_expt_access_fault_with_page | output | 1 | 246 | AG 生成的异常条件。 |
| ld_ag_expt_ldamo_not_ca | output | 1 | 247 | 原子 load/AMO 访问 non-cacheable 页的专用异常。 |
| ld_ag_expt_misalign_no_page | output | 1 | 248 | AG 生成的异常条件。 |
| ld_ag_expt_misalign_with_page | output | 1 | 249 | AG 生成的异常条件。 |
| ld_ag_expt_page_fault | output | 1 | 250 | AG 生成的异常条件。 |
| ld_ag_expt_vld | output | 1 | 251 | 聚合异常：misalign、SO misalign、带页属性 access fault、page fault。 |
| ld_ag_iid | output | 6:0 | 252 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_inst_type | output | 1:0 | 253 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_inst_vfls | output | 1 | 254 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_inst_vld | output | 1 | 255 | AG 当前指令有效；stall 时保持，flush/reset 时清零。 |
| ld_ag_ldfifo_pc | output | 14:0 | 256 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_lm_init_vld | output | 1 | 257 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_lr_inst | output | 1 | 258 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_lsid | output | 11:0 | 259 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_lsiq_bkpta_data | output | 1 | 260 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_lsiq_bkptb_data | output | 1 | 261 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_lsiq_spec_fail | output | 1 | 262 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_no_spec | output | 1 | 263 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_no_spec_exist | output | 1 | 264 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_old | output | 1 | 265 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_pa | output | 39:0 | 266 | MMU PPN 与最终 VA 低 12 位拼接的物理地址。 |
| ld_ag_page_buf | output | 1 | 267 | MMU 页面属性的 AG 侧输出。 |
| ld_ag_page_ca | output | 1 | 268 | MMU 页面属性的 AG 侧输出。 |
| ld_ag_page_sec | output | 1 | 269 | MMU 页面属性的 AG 侧输出。 |
| ld_ag_page_share | output | 1 | 270 | MMU 页面属性的 AG 侧输出。 |
| ld_ag_page_so | output | 1 | 271 | MMU 页面属性的 AG 侧输出。 |
| ld_ag_pf_inst | output | 1 | 272 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_preg | output | 6:0 | 273 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_raw_new | output | 1 | 274 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_secd | output | 1 | 275 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_sign_extend | output | 1 | 276 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_split | output | 1 | 277 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_stall_ori | output | 1 | 278 | 不含 atomic-no-commit restart 的原始 stall 指示。 |
| ld_ag_stall_restart_entry | output | 11:0 | 279 | stall/restart 时选中的 LSIQ entry bitmap。 |
| ld_ag_utlb_miss | output | 1 | 280 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_vpn | output | 27:0 | 281 | load AG stage 的状态、地址、分类或控制输出。 |
| ld_ag_vreg | output | 5:0 | 282 | load AG stage 的状态、地址、分类或控制输出。 |
| lsu_hpcp_ld_cross_4k_stall | output | 1 | 283 | 模块接口信号；具体有效条件由相邻控制信号和功能规则限定。 |
| lsu_hpcp_ld_other_stall | output | 1 | 284 | 模块接口信号；具体有效条件由相邻控制信号和功能规则限定。 |
| lsu_idu_ag_pipe3_load_inst_vld | output | 1 | 285 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_preg_dup0 | output | 6:0 | 286 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_preg_dup1 | output | 6:0 | 287 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_preg_dup2 | output | 6:0 | 288 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_preg_dup3 | output | 6:0 | 289 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_preg_dup4 | output | 6:0 | 290 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_vload_inst_vld | output | 1 | 291 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_vreg_dup0 | output | 6:0 | 292 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_vreg_dup1 | output | 6:0 | 293 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_vreg_dup2 | output | 6:0 | 294 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ag_pipe3_vreg_dup3 | output | 6:0 | 295 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ld_ag_wait_old | output | 11:0 | 296 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |
| lsu_idu_ld_ag_wait_old_gateclk_en | output | 1 | 297 | 从 LSU/AG 返回 IDU 的调度或目的寄存器信息。 |

# 5. Functional Requirements

| ID | Requirement | Source | Normative behavior | Verification |
| --- | --- | --- | --- | --- |
| FR-01 | AG valid capture | L582-L591 | Reset or flush clears valid. Otherwise stall or RF select sets valid; no stall and no RF select clears valid. | Check all priority combinations, including flush with stall and flush with new issue. |
| FR-02 | AG data capture | L626-L692 | When no AG stall and RF data is valid, all listed instruction attributes are captured. During stall, they retain their previous values. | Randomize every field and compare cycle-accurate hold/load behavior. |
| FR-03 | Immediate/register offset | L728-L765 | Normal load uses sign-extended 12-bit immediate; LDR uses src1 with optional high zero extension; split/replay may replace the offset. | Cross instruction type, sign, zero-extend, and replay. |
| FR-04 | Offset shift | L713-L720, L789-L792 | shift is interpreted as one-hot select of offset << 0,1,2,3. | Verify four legal values and inject all illegal encodings. |
| FR-05 | VA selection | L794-L808 | va_ori = base + shifted_offset. A first, boundary, ordinary load that is not LDR selects va_plus; otherwise va_ori is selected. | Reference-model 64-bit modular arithmetic and split behavior. |
| FR-06 | Instruction classes | L816-L835 | Ordinary load, LR, LDAMO, prefetch, scalar/vector-FLS, and tied-off feature classifications follow the explicit equations. | Enumerate atomic x inst_type x split x lsfifo x secd. |
| FR-07 | Access size | L842-L868 | BYTE/HALF/WORD/DWORD map to size-minus-one 0/1/3/7 and DC size codes 0/1/2/3. | Exhaustive four-code check. |
| FR-08 | Natural alignment | L872-L900 | BYTE is always aligned; HALF requires A0=0; WORD A1:0=0; DWORD A2:0=0. SO uses the same alignment result. | Sweep address low bits for every size. |
| FR-09 | 16-byte boundary | L903-L909 | Boundary is detected by adding start offset and size-minus-one. Output boundary is restricted to ordinary load and remains asserted on secd. | Sweep all 16 starts and four sizes; verify first/second beat. |
| FR-10 | Byte masks | L915-L985 | Masks identify bytes from start to end within a 16-byte window; secd selects the high-side mask. | Compare against a software mask model for all starts/sizes/secd. |
| FR-11 | MMU request/abort | L1035-L1044 | MMU request valid follows AG valid. Address is base. Abort asserts for page replay, atomic wait, cache stall, no-page misalign, or flush. | Check each abort cause alone and in priority combinations. |
| FR-12 | PA and attributes | L1047-L1068 | PA combines MMU PPN with selected VA low 12 bits. CA/SO are valid-gated; other attributes pass through. | Use random PPN/VA and invalid MMU cycles. |
| FR-13 | Boundary acceleration | L1071-L1083 | Acceleration requires boundary, same 4 KiB page, feature enable, first beat, dcache enable, and no prior replay; DC enable additionally requires PA valid/cacheable. | Cross every qualifier and check load classification. |
| FR-14 | Commit match | L1109-L1119 | Current iid is considered committed when any of three valid commit ports carries the same iid. | Match each port and simultaneous/nonmatching combinations. |
| FR-15 | DCACHE array request | L1124-L1190 | Tag/data array requests are speculative when AG valid and dcache enabled. Bank enables cover 32-bit banks touched by the access; acceleration selects all banks. | Sweep start/end banks, boundary/secd, dcache enable, and MMU invalid. |
| FR-16 | Exceptions | L1204-L1241 | Generate no-page misalign, SO misalign, page fault, LDAMO non-CA, and vector-SO access fault per RTL equations. Aggregate expt_vld excludes LDAMO non-CA. | Directed test each exception plus simultaneous conditions and priority at later stages. |
| FR-17 | Atomic restart | L1247-L1249 | Valid atomic instruction restarts until a matching RTU commit is observed. | Delay commit, use wrong iid, then matching iid; verify hold and release. |
| FR-18 | 4 KiB crossing | L1256-L1282 | Crossing is low-12 carry or illegal high offset extension. Replay is suppressed by no-page misalign. | Boundary-focused random and formal arithmetic equivalence. |
| FR-19 | Stall priority | L1287-L1316 | Priority is atomic wait, cross-page/LDR replay, DCACHE stall, then MMU stall. Older RF instruction may mask non-atomic stall. | Pairwise and all-cause tests with iid age wrap cases. |
| FR-20 | DC advance | L1342-L1350 | DC valid requires AG valid and no restart/stall. Borrow address overrides normal PA when borrow valid. | Check every stall cause and borrow mux. |
| FR-21 | Ahead signaling | L1357-L1382, L1423-L1438 | Ahead prediction is constant 1. Scalar/vector valid is qualified by class, boundary, forwarding-disable, and tied-off vector features. | Prove cancel/replay for cache/TLB/exception misses. |
| FR-22 | Performance events | L1447-L1456 | Cross-4K and other-stall counters are qualified by valid and exclude uTLB miss/already-DA as defined. | Generate isolated and overlapping stall classes and count pulses. |

# 6. Configuration Matrix

| Configuration | Values | Effect | Required test |
| --- | --- | --- | --- |
| cp0_lsu_dcache_en | 0/1 | Enables speculative DCACHE tag/data requests and boundary acceleration precondition. | Both values; toggle only between transactions unless design confirms dynamic behavior. |
| cp0_lsu_cb_aclr_dis | 0/1 | Disables 16-byte boundary acceleration when 1. | Cross with boundary, PA valid, cacheable, and secd. |
| cp0_lsu_da_fwd_dis | 0/1 | Suppresses scalar/vector ahead outputs when 1. | Toggle for ordinary and vector-FLS load. |
| cp0_lsu_mm | 0/1 | Controls whether ordinary-load misalignment is reported without waiting for page attributes. | Cross with alignment, atomic, SO, and PA valid. |
| cp0_yy_clk_en | 0/1 | Global clock enable for the AG data clock. | Clock-gating and wake-up test. |
| cp0_lsu_icg_en | 0/1 | Module-level ICG enable. | Clock-gating and scan override test. |
| pad_yy_icg_scan_en | 0/1 | Scan-mode ICG override. | DFT-mode clock propagation test. |

# 7. Verification Strategy

## 7.1 Reference Model

The scoreboard shall model 64-bit modular address arithmetic, size-minus-one, natural alignment, 16-byte masks, 4 KiB crossing, and PA concatenation. Protocol state shall include AG valid/data hold, already-crossed replay state, atomic commit wait, and older-RF stall masking.

## 7.2 Directed Test Set

| Test | Name | Stimulus and checks |
| --- | --- | --- |
| T01 | Reset/idle X | Release reset with no issue; verify no architectural request/event is accepted despite undefined data fields. |
| T02 | Issue and advance | Single ordinary load, MMU hit, DCACHE selected; verify one-cycle AG residency and DC valid. |
| T03 | Flush priority | Assert flush with new issue and with each stall cause; valid shall clear. |
| T04 | Data hold | Random data fields while stalled; all AG registers shall remain stable except defined replay fields. |
| T05 | Address arithmetic | Positive/negative immediate, LDR src1, zero extension, four shift values, wraparound. |
| T06 | Alignment sweep | All size codes x VA[3:0]. |
| T07 | Mask sweep | All size codes x VA[3:0] x secd; compare 16-bit masks and rot_sel. |
| T08 | Bank sweep | All start/end 4-byte banks, boundary and acceleration states. |
| T09 | 4 KiB replay | Addresses around page end with positive/negative offsets and split/LDR cases. |
| T10 | Atomic commit | No commit, wrong iid, each commit port match, simultaneous matches, flush. |
| T11 | MMU miss/fault | pa_vld 0, page fault, CA/SO/BUF/SEC/SH attributes, abort causes. |
| T12 | Non-cacheable AMO | Verify dedicated exception and zero architectural side effects. |
| T13 | Boundary acceleration | Enable/disable, cacheable/non-cacheable, PA valid/invalid, SO, 4 KiB cross. |
| T14 | DCACHE backpressure | Toggle arb select; verify hold, abort, and no duplicate DC transaction. |
| T15 | Older RF mask | Exercise iid age comparator including wrap boundary; verify stall entry selection. |
| T16 | Ahead cancellation | Cache miss, uTLB miss, page fault, boundary, flush, forwarding disable. |
| T17 | Illegal shift | Inject zero-hot and multi-hot encodings; expect assertion or documented safe behavior. |
| T18 | vreg bit 6 | Drive bit 6 both ways and validate the intended six-bit contract. |
| T19 | Idle MMU stall | Force stall0 during AG idle; verify contract/assertion prevents phantom valid. |
| T20 | Clock gating | Local/global/module/scan enable transitions with issue, stall, and reset. |

## 7.3 Constrained-Random

Random instruction attributes with legal one-hot shift and valid size/type constraints.

Random MMU latency/result and DCACHE arbitration, while respecting or deliberately violating documented assumptions in negative tests.

Random RTU commit timing/iid for atomic requests.

Random flush at every AG state and replay phase.

Scoreboard every accepted RF transaction through advance, restart, exception, or flush termination.

# 8. Suggested Assertions

| ID | Intent | Property sketch |
| --- | --- | --- |
| A01 | Flush clears valid | rtu_yy_xx_flush ¦=> !ld_ag_inst_vld |
| A02 | Stall holds instruction identity | ld_ag_stall_vld && !rtu_yy_xx_flush ¦=> $stable(ld_ag_iid) |
| A03 | DC valid implies AG valid | ld_ag_dc_inst_vld ¦-> ld_ag_inst_vld |
| A04 | DC valid excludes restart | ld_ag_dc_inst_vld ¦-> !ld_ag_stall_restart |
| A05 | MMU request valid equality | lsu_mmu_va0_vld == ld_ag_inst_vld |
| A06 | Boundary only ordinary load | ld_ag_boundary ¦-> ld_ag_ld_inst |
| A07 | Acceleration legality | ld_ag_dc_acclr_en ¦-> mmu_lsu_pa0_vld && ld_ag_page_ca && ld_ag_boundary |
| A08 | Legal shift | RF accept ¦-> $onehot(idu_lsu_rf_pipe3_shift) |
| A09 | No idle MMU stall | !ld_ag_inst_vld && !idu_lsu_rf_pipe3_sel ¦-> !mmu_lsu_stall0 |
| A10 | vreg high bit contract | RF vector accept ¦-> !idu_lsu_rf_pipe3_vreg[6] |
| A11 | Array request qualification | ag_dcache_arb_ld_tag_req ¦-> ld_ag_inst_vld && cp0_lsu_dcache_en |
| A12 | Mask nonzero for valid size | ld_ag_inst_vld ¦-> ld_ag_dc_bytes_vld != 16'h0000 |

Property sketches are not drop-in SVA. Bind paths, sampling clocks, reset conditions, and helper definitions shall be adapted to the verification environment.

# 9. Coverage Model

| Covergroup | Coverpoints |
| --- | --- |
| Instruction | atomic, inst_type, inst_size, LDR, split, secd, FLS, sign-extend |
| Address | VA[3:0], page offset near 0/FFF, offset sign, shift one-hot, boundary, cross-4K |
| MMU | pa_vld, page_fault, CA, SO, BUF, SEC, SH, stall |
| Control | new issue, stall reason, older-RF mask, flush, commit port/hit |
| Configuration | dcache_en, aclr_dis, da_fwd_dis, mm, ICG enables |
| Outputs | mask pattern, bank enable, tag/data index, exception class, DC valid, ahead valid, HPCP event |

## 9.1 Mandatory Crosses

inst_size x VA[3:0] x boundary x secd

boundary x pa_vld x page_ca x page_so x cp0_lsu_cb_aclr_dis

cross_4K x offset_sign x shift x LDR/split

exception_class x DCACHE request x DC valid

stall_reason x atomic x commit_hit x older_RF_mask

ahead_valid x cache/TLB outcome x flush/replay

# 10. Open Questions and Assumptions

| ID | Question / required contract |
| --- | --- |
| Q1 | Is mmu_lsu_stall0 guaranteed to remain 0, or at least 0 when AG is idle? |
| Q2 | Is idu_lsu_rf_pipe3_shift strictly one-hot? What is required for illegal encodings? |
| Q3 | Is idu_lsu_rf_pipe3_vreg[6] reserved and guaranteed 0? |
| Q4 | Is ld_ag_ahead_predict = 1 the final policy, and which downstream mechanism cancels every misprediction? |
| Q5 | May DCACHE tag/data arrays be read before PA valid, cacheability, and exception resolution? Which consumer prevents side effects? |
| Q6 | Is exclusion of ld_ag_expt_ldamo_not_ca from ld_ag_expt_vld intentional for every aggregate-exception consumer? |
| Q7 | Why does boundary load-valid use ld_ag_acclr_en rather than the PA-qualified ld_ag_dc_acclr_en? |
| Q8 | Does every page-fault response assert mmu_lsu_pa0_vld in the same cycle? |
| Q9 | What exact phase and stop-clock relationship is guaranteed between ctrl_ld_clk and ld_ag_clk? |
| Q10 | Are vector mask, FOF, VMB merge, and vector-ahead functions permanently removed or planned for restoration? |

# Appendix A. Traceability

The companion ct_lsu_ld_ag.md contains one annotation row for every physical source line. The companion ct_lsu_ld_ag_risk_review.md ranks protocol and implementation risks. The Excel feature list maps verification features to RTL line ranges and test methods.

# Appendix B. Known RTL Version Characteristics

Vector mask block is absent/tied off in this version.

ld_ag_inst_vls, ld_ag_inst_fof, ld_ag_vmb_merge_vld, and ld_ag_dc_vload_ahead_inst_vld are constant 0.

ld_ag_ahead_predict is constant 1.

MMU port0 stall is constant 0 in the matching upstream OpenC910 DUTLB implementation, but not constrained inside this module.

The supplied RTL is byte-identical to the current upstream OpenC910 file at the URL on the cover page as checked on 2026-07-11.